# TECHNICAL SPECIFICATION 	
FABRIC-BACKED VINYL WALLCOVERING

______________________________________________________________

Product			Gilded Masonry

Description		Commercial quality vinyl wallcovering with mylar, laminated onto a non woven

backing.

Weight		 	450 grammes per square metre.

Roll Size		137cm x 27m

Fire Rating		Euroclass B

Adhesive		Generally Tekfix Plus, though depends on substrate. Tekfix VOV should be used when installing around right angled corners.

Hanging		Care should be taken during installation in order to produce optimum results. Apply adhesive to wall surface. Carefully smooth wallcovering onto wall surface using a spatula, allowing 5-10cm at top and bottom for trimming. Overlap lengths, and double cut through. Run seam roller up join to ensure even adhesion. Remove any adhesive seepage immediately with detergent solution and clean water. Rinse thoroughly. Do not reverse alternate lengths. Gilded Masonry has a straight match / 64cm repeat. .

Maintenance	 	Fabric-backed vinyls offer an easy-clean finish, and high resistance to knocks and scrapes.  They are designed to withstand high traffic levels, and will not tear easily.  With regular cleaning they will retain their original appearance for many years depending on area of use.

Always carry out a trial in an inconspicuous area first, to ensure cleaning agent suitable.  Clean with a mild non-soapy detergent with soft cloth, sponge or soft bristle brush to prevent build up of dirt and staining. Avoid polishes and abrasive cleaning agents as they will cause build up of dirt in the surface texture, and can also damage metallic finishes. Avoid strong active solvent cleaners as they can stain and damage the vinyl surface. Always clean from the skirting level upwards.  Rinse thoroughly with clean water.  Individual stains should be removed as quickly as possible. This wallcovering contains antimicrobials which guard against mildew and bacterial growth.

Health & Safety		This product is classified as non-hazardous and therefore does not require 				any special handling.  This product does not require special labelling.

Handling/Storage	Store at ambient temperatures away from heat source.  Suitable lifting 				equipment may be required.

Fire Fighting		Any extinguishing media are suitable.  Wear self contained breathing 				apparatus, protective clothing and full face mask.  Fumes should be 				ventilated immediately due to possible liberation of noxious gases.

Ecological		This product is not bio-degradable.  Dispose of in accordance with Local 				Authority regulations or via an authorised contractor.

__________________________________________________________________________________

TEKTURA PLC, Suite 1.03 Harbour Island, 34 Harbour Exchange Square, London E14 9GE
Tel  020 7536 3300   Fax 020 7536 3322